Low power solar 4G bullet camera           Model :LS-4GS06

Key Features：

1. No need for external electricity, solar power generation and built-in lithium battery power supply;

2. No need to connect the network cable, built-in 4G module, support 4G frequency band selection in different countries；

3. Microwave motion detection technology:Actively detection in 0.6S,Fast,longer and wider range detection,Not affected by environment, temperature and reflect light.

4. Remote viewing by mobile APP (Mycam);

5. Human body movement triggers alarm and video recording;

6. Local storage, cloud storage;

7. The installation is simple, install by yourself, no need to hire professional construction personnel;

Application scenarios：

Doors of homes, yards, fish ponds, orchards, farms, mines, construction sites, and all places where electricity and wiring are not convenient.

Specification:

| model: | LS-4GS06 |
| --- | --- |
| color: | white |
| Material | Metal |
| Induction principle: | HMD（Human Motion Detection-microwave） |
| CPU: | T21Z/T31Z |
| Sensor: | F23/2063 2mp 1/2.7" CMOS |
| 4G module: | Qualcomm SIM7600 Series/V3E |
| 4G frequency: | 4G for APN/Euro/USA/Japan etc options |
| Network: | TCP/IP，HTTP，TCP，UDP，SMTP，DHCP，DNS，P2P |
| SIM card: | support |
| Resolution | 2.0MP |
| Image size : | 1920*1080 |
| Video compressed format: | H.265 |
| Lens | 4mm |
| Detection time: | 0.6S |
| Detection distance | 12m |
| Detection angle: | 120º |
| Battery type： | 18650 Li-ion battery |
| Battery Qty： | Built in 4pcs,2600mAh/pcs |
| battery capcity: | 10400mAh |
| Solar panel power: | 5.5W（5.5V/1000mA） |
| Max Charge Current : | 1000mA |
| Night vision mode | starlight colorful and white&black combine |
| Night vision distance | 15M |
| White light power: | 1.5W LED 0.75W*2 |
| Infrared power: | 1W LED 0.5W*2 |
| Storage: | Support TF Card max 128GB |
| Working temperature: | -10℃ to 65℃ |
| Working humidity: | 0%-90%RH |
| Power supply: | solar + battery |
| Stand-by power consumption: | around 0.05W |
| Work power consumption: | daytime≤2W, night version ≤2.75W |
| Waterproof | IP66 |
| Accessories: | camera*1、braket*1、scew pack*1、manual*1、USB cable、Tweezers |
| optional accessory: | external solar panel, extension cable |
| CAM size: | 235mm*240mm*115mm |
| CAM weight: | 1.4kg |
| Gift box size: | 262*136*275mm |
| G.W/PCS: | 1.9kg |
| Carton size: | 695x535×295mm |
| G.W/CTN: | 21kg |
| Qty/CTN: | 10pcs /case |